HW/SW Co-Design Project

Benchmark report: nbody

pyperformance optimization and hardware acceleration (V3A)

| Software speedup | Run time | Correctness | Hardware (V3A) |
| --- | --- | --- | --- |
| 1.64x | 229 ms -> 140 ms | energy diff 2.2e-16 | 38 cycles/step, ~18x |

Environment: Linux-5.15.0-1106-kvm-x86_64-with-glibc2.35, 1 CPU(s), Python 3.10.12 (64-bit), pyperformance 1.14.0.

# 1. Overview

Purpose

nbody comes from the Computer Language Benchmarks Game. It simulates the

orbits of the Jovian planets (Sun, Jupiter, Saturn, Uranus, Neptune) under

Newtonian gravity. It is a classic "pure floating-point number crunching"

workload: no I/O, no allocation of user objects, no strings -- only

arithmetic on Python floats stored in Python lists. It therefore measures

how fast the CPython interpreter executes numeric bytecode.

Libraries

- Python standard library only. No numpy / C extensions.

- pyperf: timing harness only (perf_counter, worker processes, statistics);

it is not part of the measured computation.

Data structures

- BODIES : dict name -> (position, velocity, mass)

position = [x, y, z]      (mutable list of 3 floats)

velocity = [vx, vy, vz]   (mutable list of 3 floats)

mass     = float          (in units where G = 1, SOLAR_MASS = 4*pi^2)

- SYSTEM : list of the 5 body tuples (the lists inside are shared by

reference, so in-place updates are visible everywhere).

- PAIRS  : list of the 10 unordered body pairs, built once by a pure-Python

re-implementation of itertools.combinations(SYSTEM, 2).

Algorithm

Direct all-pairs O(N^2) gravity with a semi-implicit (symplectic) Euler

integrator, dt = 0.01:

for each step:

for each pair (i, j):

d   = r_i - r_j

mag = dt * |d|^-3

v_i -= d * m_j * mag

v_j += d * m_i * mag

for each body:

r   += dt * v              (uses the NEW velocity -> symplectic)

report_energy() computes total energy (kinetic + potential) before and after

the run as a sanity check; offset_momentum() makes the total momentum zero

by adjusting the Sun's velocity.

One benchmark "loop" = report_energy + 20,000 steps + report_energy.

Per step: 10 pair interactions + 5 position updates, i.e. 200,000 pair

interactions per loop. advance() is >99.9% of the work: report_energy()

evaluates the 10 pairs once, advance() evaluates them 20,000 times.

Why this is slow in CPython

A float in CPython is a heap object (PyFloatObject, 24 bytes). Every

arithmetic bytecode goes: dispatch in _PyEval_EvalFrameDefault ->

PyNumber_Multiply -> float_mul -> PyFloat_FromDouble (free-list allocation)

-> Py_DECREF of temporaries. The actual double multiply is ~1 machine

instruction; the surrounding interpreter work is tens to hundreds.

In addition, every `v1[0] -= x` needs BINARY_SUBSCR + INPLACE_SUBTRACT +

STORE_SUBSCR with stack shuffling (DUP_TOP_TWO, ROT_THREE), and every pair

iteration needs FOR_ITER and three UNPACK_SEQUENCE operations.

# 2. Initial analysis (profiling)

Method (all automated in script_nbody.sh)

- pyperformance run -b nbody                      -> results/baseline.json

- perf stat -r 5 (instructions, branches, cache)  -> results/perf_stat_original.txt

- perf record -e cpu-clock -F 999 -g -- python3-dbg -m pyperformance run

--bench nbody (course guide method)             -> results/perf_report_original*.txt

- FlameGraph (stackcollapse-perf.pl | flamegraph.pl)

-> flamegraphs/flamegraph_nbody_original.svg

- bytecode_count_nbody.py (executed opcodes/step) -> table below

python3-dbg is used so perf can resolve CPython's internal C symbols.

Python 3.10 has no `-X perf` support, so the perf flame graph shows

interpreter C functions, not Python function names.

VM note: the QEMU vCPU exposes no "cycles" event, so perf's default

sampling event records zero samples; we sample on cpu-clock instead.

perf stat runs profile_nbody.py (bench_nbody() called directly in one

process) so counters exclude pyperf worker spawning and venv setup.

Baseline result

pyperformance run -b nbody:  229 ms +- 3.0 ms per loop  (60 values)

(under python3-dbg while profiling: 481 ms +- 3.2 ms)

perf report -- hottest symbols, self overhead (original)

overhead  symbol

22.29%  _PyEval_EvalFrameDefault

2.20%  binary_op1

2.03%  validate_list

1.88%  __memset_avx2_unaligned_erms

1.86%  _PyMem_DebugCheckAddress

1.61%  PyFloat_FromDouble

1.47%  float_mul.lto_priv.0

1.46%  lookdict_unicode_nodummy

1.37%  _PyPegen_is_memoized

1.32%  float_dealloc.lto_priv.0

1.15%  read_size_t

1.05%  _PyObject_Malloc

0.98%  list_ass_item.lto_priv.0

0.98%  _PyUnicode_CheckConsistency

0.96%  PyNumber_AsSsize_t

grouped by role:

22.29%  bytecode dispatch (_PyEval_EvalFrameDefault)

7.14%  list/tuple indexing (v[0], unpacking)

6.95%  float objects: arithmetic + alloc/free

0.64%  pow() in libm

4.27%  number-protocol dispatch (binary_op1 ...)

4.75%  object alloc / refcount bookkeeping

9.98%  debug-build self-checks (python3-dbg only)

2.56%  dict lookups (globals/attributes)

(98.1% of all samples are in the benchmark worker process; the rest

is pyperformance start-up. Shares below 100% in total are the long tail

of smaller symbols.)

perf stat (original), per run of 5 loops x 20,000 steps

instructions        8,713,724,770

branches            1,500,031,830

branch-misses           6,778,605   0.45% of branches

cache-references        5,427,800

cache-misses              159,132   2.93% of cache refs

task-clock              1266.7 ms

cycles           not exposed by the VM

Interpretation: cache misses are negligible (159,132 for 8.7 billion

instructions; the whole simulation state is 5 bodies, a few KB) and

branch misses are negligible (0.45%). The workload is interpreter-

overhead bound: ~87,137 machine instructions per simulation step,

i.e. ~65 x86 instructions per executed bytecode -- for

arithmetic whose real work (a double add/mul) is ~1 instruction.

Speedup must come from executing fewer bytecodes, not from memory layout.

Executed bytecode per simulation step (original)

LOAD_FAST                397.0

STORE_FAST               186.0

BINARY_MULTIPLY          135.0

LOAD_CONST                85.0

DUP_TOP_TWO               75.0

BINARY_SUBSCR             75.0

ROT_THREE                 75.0

STORE_SUBSCR              75.0

UNPACK_SEQUENCE           60.0

INPLACE_ADD               45.0

BINARY_SUBTRACT           30.0

INPLACE_SUBTRACT          30.0

BINARY_ADD                20.0

FOR_ITER                  18.0

JUMP_ABSOLUTE             16.0

BINARY_POWER              10.0

GET_ITER                   2.0

TOTAL                   1334.1

Bottlenecks identified

B1  Container access: 150 list subscripts + 150 stack-shuffle opcodes per

step exist only because body state lives inside lists.

B2  Loop/unpacking overhead: FOR_ITER + 60 UNPACK_SEQUENCE per step to walk

PAIRS and destructure ((pos, vel, m), (pos, vel, m)).

B3  Redundant arithmetic: `dt * ...` is recomputed for every pair although

dt and masses are constant during the run.

B4  Boxed-float arithmetic itself (~270 arithmetic opcodes per step, each

allocating a new PyFloat). This is the irreducible cost in pure Python

and the target of the hardware proposal (section 5).

Flame graph - nbody, original (perf record -e cpu-clock -g, python3-dbg). Interactive version: benchmarks/nbody/flamegraphs/flamegraph_nbody_original.svg

# 3. Optimizations

All changes are in benchmarks/nbody/optimized/run_benchmark.py. The public

API (advance, report_energy, offset_momentum, bench_nbody), the workload

(5 bodies, 20,000 steps, dt = 0.01) and the benchmark name are unchanged, so

it runs under pyperformance through a custom manifest

(benchmarks/nbody/manifest/MANIFEST) and compares directly with the original.

O1  Precompute m*dt  (targets B3)

mdt_k = m_k * dt is computed once per call. The pair loop then uses

mag = |d|^-1.5 and b1m = mdt_i * mag -> one fewer multiply per pair.

O2  Body state in local variables  (targets B1)

At the start of advance(), positions/velocities are unpacked into 30

local variables (x0..z4, vx0..vz4); they are written back to the lists

once at the end. Locals live in the frame's fastlocals array, so access

is LOAD_FAST/STORE_FAST (an array index) instead of list subscripting.

O3  Specialization / loop unrolling  (targets B2)

Since the number of bodies is fixed during a run, the 10 pair

interactions are generated as straight-line code. The generator runs once

at import time for len(SYSTEM) bodies (so the code is general for any N)

and is compiled with compile()/exec(); nothing is generated in the timed

region. `python3 run_benchmark.py --dump-generated` prints it. This is

the same idea a JIT uses (specializing code for values that are constant

at run time). O2 is only possible together with O3: without unrolling

there is no way to name "the velocity of body j" as a local variable.

Correctness (verify_nbody.py, 20,000 steps from identical initial state)

steps=20000

energy before : original=-0.1690751638285245  optimized=-0.1690751638285245

energy after  : original=-0.16908926275527172  optimized=-0.16908926275527195

|dE|=2.220e-16   max |state diff|=6.117e-12

PASS

A ~1e-16 energy difference is expected: O1 computes (m*dt)*mag instead

of m*(dt*mag), and IEEE-754 multiplication is not associative.

Effect on executed bytecode (per step)

opcode                 original  optimized

LOAD_FAST                 397.0      385.8

STORE_FAST                186.0      136.6

BINARY_MULTIPLY           135.0      125.0

LOAD_CONST                 85.0       10.4

INPLACE_ADD                45.0       45.0

STORE_SUBSCR               75.0        0.3

BINARY_SUBSCR              75.0        0.1

ROT_THREE                  75.0        0.0

DUP_TOP_TWO                75.0        0.0

UNPACK_SEQUENCE            60.0        0.2

INPLACE_SUBTRACT           30.0       30.0

BINARY_SUBTRACT            30.0       30.0

BINARY_ADD                 20.0       20.0

BINARY_POWER               10.0       10.0

FOR_ITER                   18.0        0.0

JUMP_ABSOLUTE              16.0        1.0

GET_ITER                    2.0        0.0

EXTENDED_ARG                0.0        1.0

TOTAL                    1334.1      795.5   (-40%)

Container-access, stack-shuffle and unpacking opcodes disappear; the

float arithmetic opcodes remain (minus the dt multiplies removed by O1).

Rejected / negative results (kept in ablation_nbody.py on purpose)

R1  Replace x**-1.5 with 1/(d2*sqrt(d2)). Common advice in C, but in CPython

it saves a libm pow() call while adding a CALL_FUNCTION to math.sqrt

plus an extra division opcode -- interpreter overhead cancels the win

(see ablation table).

R2  numpy vectorization. With only 5 bodies / 10 pairs per step, each numpy

call's fixed overhead (argument parsing, array allocation, dtype

dispatch) dominates the tiny amount of actual arithmetic; numpy only

wins when N is large. This is the same "offload granularity" trade-off

that applies to the hardware accelerator in section 5.

# 4. Performance comparison

pyperformance (results/compare.txt)

original : 229 ms +- 3.0 ms

optimized: 140 ms +- 4.9 ms

-> 1.64x faster, run time -39%  (meets the >= 7% requirement)

Same trend under python3-dbg while profiling: 481 ms -> 274 ms (1.76x).

Ablation (ablation_nbody.py) -- each optimization measured separately

variant                       time / 20k steps  vs original

original                         228.7 +-   3 ms      1.00x

R1 sqrt instead of pow           229.4 +-   3 ms      1.00x

O1 precompute m*dt               226.0 +-   4 ms      1.01x

O1+O2+O3 (final)                 139.3 +-   4 ms      1.64x

(R2 numpy skipped: numpy is not installed in this environment)

perf stat before/after (5 loops x 20,000 steps per run, mean of 5 runs)

original       optimized   change

instructions         8,713,724,770   5,568,251,540   -36.1%

branches             1,500,031,830     961,259,560   -35.9%

branch-misses            6,778,605       5,315,149   -21.6%

cache-misses               159,132          24,773   -84.4%

task-clock               1266.7 ms        789.5 ms   1.60x faster

elapsed                    1.657 s         1.108 s   1.50x faster

Instructions drop by 36% (~87,137 -> ~55,683 per step), in line with the

40% drop in executed bytecodes (1334 -> 795 per step). The time ratio

(1.60x) tracks the instruction ratio (1.56x): the gain is "less

interpreter work", not better cache or branch behaviour.

perf profile before/after (flame graphs: flamegraphs/flamegraph_nbody_*.svg)

Estimated absolute cost = self% x total sampled time of each profile.

role                                            original  optimized   change

bytecode dispatch (_PyEval_EvalFrameDefault)       22.3%      26.1%     -49%

list/tuple indexing (v[0], unpacking)               7.1%       1.1%     -93%

float objects: arithmetic + alloc/free              7.0%      13.5%     -15%

pow() in libm                                       0.6%       1.6%      +8%

number-protocol dispatch (binary_op1 ...)           4.3%      10.2%      +4%

object alloc / refcount bookkeeping                 4.7%       5.1%     -54%

debug-build self-checks (python3-dbg only)         10.0%       8.4%     -64%

dict lookups (globals/attributes)                   2.6%       2.8%     -52%

Largest absolute reductions: _PyEval_EvalFrameDefault (-49%),

__memset_avx2_unaligned_erms (-76%), _PyPegen_is_memoized (-100%),

_PyMem_DebugCheckAddress (-72%), list_ass_item.lto_priv.0 (-100%).

Practically gone from the optimized profile: list_ass_item.lto_priv.0,

_PyPegen_is_memoized, PyNumber_AsSsize_t.

Top of the optimized profile: _PyEval_EvalFrameDefault (26.1%),

binary_op1 (5.2%), PyFloat_FromDouble (3.4%), float_mul.lto_priv.0

(2.9%), float_dealloc.lto_priv.0 (2.5%).

Total sampled time: original 42.18 s, optimized 18.30 s (cpu-clock

event count, includes pyperformance start-up).

Flame graph - nbody, BEFORE optimization

Flame graph - nbody, AFTER optimization. Interactive version: benchmarks/nbody/flamegraphs/flamegraph_nbody_optimized.svg

# 5. Hardware acceleration proposal

One accelerator serves both benchmarks: V3A, a 3D Vector-math Accelerator.

The hot arithmetic of nbody (pair distance, |d|^-3) and of raytrace

(ray-sphere test: dot products and a square root) is the same small set of

operations: 3D subtract, dot product and inverse square root. V3A implements

them once as reusable pipelined IP and exposes two kernels on one PCIe card.

Full RTL: hw/rtl/*.v (Verilog-2005/SystemVerilog-compatible, Icarus-tested).

## 5.1  Hardware description (hw/rtl)

v3a_top.v             PCIe core logic: register bus decode, IRQ

v3a_nbody_ctrl.v      nbody kernel: body registers, step FSM, integrator

v3a_pair_pipe.v       pairwise gravity datapath, 19 stages

v3a_ray_ctrl.v        ray kernel: ray BRAM, feed FSM, reducer, results

v3a_raysphere_pipe.v  ray-sphere datapath, 19 stages

v3a_invsqrt.v         shared inverse-square-root IP, 12 stages

v3a_delay.v           sideband pipeline balancing

v3a_common.vh         Q23.40 fixed-point multiply

v3a_invsqrt_lut.vh    64-entry initial-guess ROM (generated)

Golden model: hw/model/v3a_model.py (bit-accurate). Testbench:

hw/tb/tb_v3a.v. Everything is run by: python3 hw/run_hw_tests.py

## 5.2  Inputs and outputs

Number format : signed Q23.40, 64 bits (range +-8.4e6, LSB 9.1e-13).

Chosen instead of IEEE-754 double: a fixed-point multiply

needs no normalization/rounding logic (about half the area

of a double-precision FPU multiplier, one stage per op).

Clock         : single clock, target 100 MHz (see 5.7), sync. reset.

Host bus      : memory-mapped, word-addressed, 64-bit data (PCIe BAR0

behind an AXI4-Lite bridge; BAR byte offset = 8 x index).

24-bit address: 0x000000 nbody window, 0x100000 ray window.

Interrupt     : irq (level) when a job is done -> MSI.

nbody kernel  : in : NBODIES (2..8), NSTEPS, DT, per body x,y,z,vx,vy,vz,m*dt

out: updated x,y,z,vx,vy,vz after NSTEPS steps; STEPDONE,

CYCLES (performance counter)

ray kernel    : in : NRAYS (1..4096), NSPH (1..16), EPS, MODE

(0 closest hit, 1 any hit), spheres c,r^2,

rays p,d (d normalized)

out: per ray (sphere index or -1, t)  /  (occluded 1/0)

Pipeline IP   : valid/data streaming, no back-pressure needed (the

controllers always accept every output).

v3a_invsqrt        in  x (Q23.40)                     out y = x^-1/2

v3a_pair_pipe      in  r_i, r_j, m_i*dt, m_j*dt, i, j out f_i, f_j, i, j

v3a_raysphere_pipe in  p, d, c, r^2, ray id, sph id   out disc, t, ids

## 5.3  Hardware architecture

Datapath (see hw/docs/v3a_pipelines.png):

- Every stage has at most ONE 64x64 multiply followed by a register, so

the critical path is the same everywhere and throughput is one result

per clock cycle after the 19-cycle latency.

- v3a_invsqrt: stage 1 normalizes x = m*4^k with m in [1,4) using a

priority encoder and a barrel shift; stage 2 takes y0 from a 64-entry

ROM indexed by the top 6 bits of m; stages 3-11 run three Newton

iterations y <- y(3 - m y^2)/2 (3 stages each: y^2, m*y^2, update);

stage 12 shifts back by k. Measured accuracy: relative error < 2.3e-7

over 1e-6..1e6 (limited by the Q23.40 LSB), ~1e-12 for typical values.

- Sideband data (indices, d, masses, v) travel through v3a_delay shift

registers so they stay aligned with the invsqrt result.

Control logic:

- v3a_nbody_ctrl FSM per time step: CLR (zero accumulators) -> FEED (one

pair (i<j) per cycle into the pipeline) -> WAIT (outputs accumulate

acc_i -= f_i, acc_j += f_j) -> VEL (v += acc for all bodies) -> POS

(r += dt*v, one body per cycle) -> NEXT (repeat NSTEPS times -> IRQ).

All pairs of a step use the positions from the start of the step, so

pairs are independent and can be streamed through the pipeline.

- v3a_ray_ctrl: a feed FSM walks (ray r, sphere s) and streams one test

per cycle (ray words come from a synchronous BRAM, 1-cycle read); a

reducer after the pipeline keeps the running best hit of the current

ray (t > -EPS and smallest t; or any t > EPS for shadow rays) and writes

the result BRAM when the last sphere of that ray comes out.

Verification (hw/run_hw_tests.py, vectors from the real benchmark code):

nbody  : 5 bodies, 2000 steps, 10 pairs/step

RTL vs golden fixed-point model: BIT-EXACT

energy: float benchmark -0.169071606870, hardware -0.169071606646

relative difference 1.33e-09

max |position diff| vs float benchmark: 9.01e-07

cycles: 76000 = 38.0 per step (10 pair feeds +

19 latency + 5 position updates + control)

ray    : closest hit, primary rays

2500 rays x 7 spheres = 17500 tests

RTL vs golden fixed-point model: BIT-EXACT

closest sphere == float benchmark: 2500/2500 rays

(194 hits), max |t error| 1.12e-09

cycles: 17521 = 1.001 per test (+19 latency, +1 BRAM read)

ray    : any hit, shadow rays

1940 rays x 7 spheres = 13580 tests

RTL vs golden fixed-point model: BIT-EXACT

shadow decision == float benchmark: 1940/1940 rays

(53 occluded)

cycles: 13601 = 1.002 per test (+19 latency, +1 BRAM read)

RESULT: PASS

## 5.4  Hardware/software interface

Driver    : Linux PCIe driver, character device /dev/v3a. mmap() of BAR0

for the registers; pinned DMA buffers for bulk data; MSI

interrupt wakes the waiting process.

Transfers : small state (nbody: 5 bodies x 7 words = 280 bytes) is written

directly through BAR0. Ray batches (48 bytes per ray, up to

4096 rays) are copied by the card's AXI4 DMA master from a

pinned host buffer into the ray BRAM; results are copied back

the same way (the RTL models the BRAM filled via the bus).

API       : Python extension module (C, or ctypes over the ioctl):

v3a.nbody_advance(bodies, dt, steps)

v3a.closest_hits(rays, spheres, eps) -> [(index, t), ...]

v3a.any_hits(rays, spheres, eps)     -> [occluded, ...]

It converts floats <-> Q23.40, fills buffers, writes CTRL,

sleeps until the IRQ and reads the results.

Key rule  : offload granularity. One PCIe round trip costs tens of

microseconds, while one CPython pair/intersection costs

~0.4-0.7 microseconds. Calling the card per pair or per ray

would be 10-100x SLOWER than software. Every job is therefore

a batch: all NSTEPS of nbody in one call; all rays of one

recursion level ("wavefront") in one call for raytrace.

(Same lesson as numpy in section 3: fixed per-call overhead

must be amortized over many elements.)

## 5.5  Acceleration justification (nbody)

Why a good candidate: advance() is >99.9% of the benchmark run time and is

a small, regular, data-independent loop of float arithmetic (no branches,

no allocation, tiny state). After the software optimization the remaining

cost is boxed-float arithmetic, which no Python change can remove, but

which maps 1:1 onto a fixed-point datapath.

Software changes: advance(dt, n) calls v3a.nbody_advance() once with the

whole state; report_energy() and offset_momentum() stay in Python.

Estimate (assumptions: 100 MHz, measured 38 cycles per step for 5 bodies

in RTL simulation, PCIe setup + 2 x 280-byte transfers + IRQ < 0.1 ms):

hardware time per loop = 20,000 steps x 38 cycles x 10 ns  = 7.6 ms

CPython original  : ~229 ms / loop  -> ~30x faster

CPython optimized : ~140 ms / loop  -> ~18x faster

(CPython numbers from results/baseline.json and optimized.json on the

course VM; the whole benchmark loop speeds up by almost the same factor,

because advance() dominates.)

Scaling: cycles per step = N(N-1)/2 + N + ~23. With N = 5 the 19-cycle

latency dominates; for N = 100 bodies: 5,073 cycles = 51 us per step vs

~3.5 ms for optimized CPython (~68x). Replicating the pair pipeline P

times divides the pair part by P (pairs within a step are independent).

Honest limit: against compiled C on a modern core (~20-50 ns per pair)

a single pipeline at 100 MHz (10 ns per pair) is only a few times faster;

the large factors above are relative to the CPython interpreter.

Accuracy: RTL results are bit-exact to the golden model; the fixed-point

trajectory stays within ~1e-6 of the double-precision benchmark after

2,000 steps and the total energy differs by ~1e-9 (relative).

## 5.6  Block diagram

Figure 1 - V3A accelerator: system integration (host, driver, PCIe, both kernels)

Figure 2 - V3A datapaths: nbody pair kernel, ray-sphere kernel, shared inverse-sqrt IP

hw/docs/v3a_system.png    system: host, driver, PCIe, both kernels

hw/docs/v3a_pipelines.png datapaths (pair pipeline and invsqrt stages)

Python advance() --> v3a ext --> driver --MMIO/DMA--> PCIe EP --> decoder

^                                     |                        |

|                                    MSI                       v

+---- updated state <--- body regs <--+-- nbody FSM --> pair pipe

(x,v,m*dt)       ^   |  (19 st.) |

^           |   v           |

+--- acc += f_i, f_j <------+

## 5.7  Performance / area / power trade-offs

Frequency: one 64x64 multiply per stage. On an FPGA (e.g. UltraScale+)

each multiply spans ~9 DSP blocks and needs ~3 internal pipeline

registers to close ~150-200 MHz; that adds latency (19 -> ~50 cycles) but

keeps 1 result/cycle. We assume a conservative 100 MHz; a 28 nm ASIC could

run ~250 MHz.

Area (order of magnitude): nbody kernel 25 multipliers (22 in the pipeline

+ 3 in the integrator), ray kernel 17 -> ~42 64x64 multipliers, ~400 DSP

blocks, ~30k flip-flops, ~44 BRAM36 for the ray buffer; in 28 nm roughly

1.5M gates (~2-3 mm^2).

Alternatives considered:

- Iterative datapath (one shared multiplier + microcode): ~20x less area,

but ~25 cycles per operation -> ~1000 cycles per nbody step, i.e. about

the SAME speed as CPython. Rejected: area saved, benefit lost.

- Fully parallel/systolic array (one pair unit per pair): 10x the area for

5 bodies, removes the 10-cycle feed time only; the 19-cycle latency

still dominates. Not worth it for N = 5, attractive for large N.

- IEEE-754 double instead of Q23.40: bit-exact with Python, but ~2x area

per multiplier and deeper pipelines; the measured fixed-point error

(~1e-9 relative energy) is far below what matters for the simulation.

- Sharing one invsqrt between both kernels: saves 9 multipliers, but the

kernels could no longer run at the same time.

Power: dominated by the DSP/multiplier toggling while busy (a few watts on

an FPGA card at 100 MHz); clock gating when idle (STATUS.busy = 0) makes

average power proportional to use. Energy per nbody loop: ~7.6 ms x a few

W ~ tens of mJ, vs ~139 ms x tens of W for a CPU core running CPython.

## 5.8  Known simplifications of this RTL (deliberate, and what a real card adds)

- Clock domains: the design uses one clock. A real PCIe card runs the user

logic on its own clock and needs asynchronous FIFOs between the PCIe

core clock and the kernel clock; that CDC logic is not modelled here.

- DMA: the RTL fills the ray BRAM through the register bus (the testbench

acts as the driver). A real card uses an AXI4 master (XDMA/QDMA IP) to

read the pinned host buffer; the register map and the FSMs do not change.

- Ray batch size: the ray BRAM holds 4096 rays, while a 100x100 frame has

10,000 primary rays, so the driver issues ~3 batches per wavefront level.

This is already included in the per-batch overhead of the estimate.

- Fixed-point overflow is not detected: Q23.40 wraps silently above

+-8.4e6. Values in both benchmarks stay far below that (the largest

intermediate is |d|^2 < 3e3), but a production version would either

saturate or raise a status flag, at the cost of extra logic per stage.

- nbody body count is limited to NMAX = 8 (state lives in registers so

every pair read is single-cycle). More bodies would move the state into

BRAM and add a load stage to the pipeline.

- No synthesis or timing closure was run (not required): the frequency

numbers are estimates based on one 64x64 multiply per pipeline stage.

# 6. Conclusion

The nbody benchmark is interpreter-overhead bound, not memory- or

branch-bound. Restructuring the hot loop so that body state lives in local

variables and the fixed set of pair interactions is unrolled removes

40% of the executed bytecodes and gives a 1.64x speedup

under pyperformance with identical physics. What remains is boxed float

arithmetic, which no pure-Python change can remove; this is exactly the

part targeted by the hardware accelerator in section 5.