HW/SW Co-Design Project

Benchmark report: raytrace

pyperformance optimization and hardware acceleration (V3A)

| Software speedup | Run time | Correctness | Hardware (V3A) |
| --- | --- | --- | --- |
| 2.85x | 797 ms -> 279 ms | bit-identical image | 1 test/cycle, ~1.7x |

Environment: Linux-5.15.0-1106-kvm-x86_64-with-glibc2.35, 1 CPU(s), Python 3.10.12 (64-bit), pyperformance 1.14.0.

# 1. Overview

Purpose

raytrace is a small, pure-Python recursive (Whitted-style) ray tracer by

Callum and Tony Garnock-Jones (2008). It renders a fixed 100x100 scene:

seven spheres over a checkerboard floor, lit by two point lights, with

specular reflections, Lambert diffuse shading with shadows, and ambient

light. It is a classic "object-oriented numeric Python" workload: millions

of tiny objects and short method calls, which stresses CPython's function

call, attribute access and allocation paths far more than the arithmetic.

Libraries

- math (sqrt, tan, pi) and array (array('B') holds the RGB canvas).

- pyperf: timing harness only; not part of the measured computation.

Data structures

- Vector, Point : classes with three float attributes x, y, z stored in a

per-instance __dict__. Vector/Point arithmetic (__add__, __sub__, scale,

dot, cross, normalized) always returns NEW objects.

- Ray           : origin Point + normalized direction Vector.

- Sphere(centre, radius), Halfspace(point, normal) : scene objects with

intersectionTime(ray) and normalAt(p).

- SimpleSurface / CheckerboardSurface : material (base colour, specular,

Lambert and ambient coefficients).

- Scene         : list of (object, surface) tuples, list of light Points,

camera; Canvas : flat array('B') of width*height*3 bytes.

Algorithm (per frame)

for each of the 10,000 pixels:

build a primary ray from the camera through the pixel (normalize)

rayColour(ray):                    (recursion depth limited to 4)

intersect the ray with ALL 8 objects, keep the closest hit t > -eps

p = hit point, n = surface normal at p

specular : reflect the ray about n and recurse

diffuse  : for each light, cast a shadow ray from p towards the light

and test it against the objects; add max(0, dir . n)

ambient  : constant term

clamp colour and write 3 bytes into the canvas

Ray-sphere test: with cp = centre - origin and v = cp . d,

discriminant = r^2 - (|cp|^2 - v^2); hit time = v - sqrt(discriminant).

Why this is slow in CPython

Every vector operation is a Python method call (frame creation, argument

binding) that allocates a new object with its own __dict__. The original

code also calls "type assertion" methods (mustBeVector(), isPoint()) that

do nothing for valid input but still cost a full call each, and it rebuilds

an identical shadow Ray (with a sqrt) for every object it tests.

# 2. Initial analysis (profiling)

Method (all automated in script_raytrace.sh)

- pyperformance run -b raytrace                   -> results/baseline.json

- perf stat -r 5 (instructions, branches, cache)  -> results/perf_stat_original.txt

- perf record -e cpu-clock -F 999 -g -- python3-dbg -m pyperformance run

--bench raytrace (course guide method)          -> results/perf_report_original*.txt

- FlameGraph (stackcollapse-perf.pl | flamegraph.pl)

-> flamegraphs/flamegraph_raytrace_original.svg

- callcount_raytrace.py (Python calls and object allocations per render)

python3-dbg lets perf resolve CPython's internal C symbols; note that

Python 3.10 has no `-X perf` support, so the flame graph shows interpreter

C functions, not Python-level functions. VM note: the QEMU vCPU exposes no "cycles" event,

so we sample on cpu-clock. perf stat runs profile_raytrace.py (5 renders in

one process) so counters exclude pyperf worker spawning and venv setup.

Baseline result

pyperformance run -b raytrace:  797 ms +- 5.8 ms per render  (60 values)

(under python3-dbg while profiling: 2175 ms +- 24.3 ms)

perf report -- hottest symbols, self overhead (original)

overhead  symbol

26.58%  _PyEval_EvalFrameDefault

3.30%  call_function.lto_priv.0

2.98%  frame_dealloc.lto_priv.0

2.72%  _PyEval_MakeFrameVector

1.98%  lookdict_unicode_nodummy

1.97%  _PyDict_GetItemHint

1.96%  PyTuple_GetItem

1.93%  binary_op1

1.88%  _PyType_Lookup

1.77%  _PyObject_VectorcallTstate.lto_priv.10

1.57%  _PyObject_GetMethod

1.57%  lookdict_split.lto_priv.0

1.53%  _PyMem_DebugCheckAddress

1.14%  _PyEval_Vector

1.12%  pthread_getspecific@@GLIBC_2.34

grouped by role:

26.58%  bytecode dispatch (_PyEval_EvalFrameDefault)

16.31%  Python function calls / frames

14.67%  attribute access (getattr/setattr, dict, slots)

5.60%  object creation / destruction

4.02%  float objects: arithmetic + alloc/free

0.00%  math: sqrt / libm

3.40%  number-protocol dispatch

3.48%  tuple/list handling

6.27%  debug-build self-checks (python3-dbg only)

(99.3% of all samples are in the benchmark worker process; the rest

is pyperformance start-up.)

perf stat (original), per run of 5 renders

instructions       25,641,141,365

branches            4,945,496,813

branch-misses          14,637,980   0.30% of branches

cache-references       13,323,843

cache-misses               34,366   0.26% of cache refs

task-clock              4061.4 ms

cycles           not exposed by the VM

Interpretation: cache misses are negligible (0.26% of cache references)

and branch misses are negligible (0.30%). One render executes ~5.13

billion instructions for 2,882,307 Python function calls, i.e. ~1,779

machine instructions per Python-level call. The cost is dominated by

call and object overhead, not by memory access or the floating-point

math itself.

Python function calls per render (callcount_raytrace.py, original)

Vector.mustBeVector                    520,539    18.1%

Vector.dot                             509,871    17.7%

Vector.__init__                        452,943    15.7%

Point.__sub__                          277,865     9.6%

Point.isPoint                          277,865     9.6%

Sphere.intersectionTime                179,457     6.2%

Vector.scale                           149,743     5.2%

Vector.normalized                      109,887     3.8%

Vector.magnitude                       109,887     3.8%

Ray.__init__                            98,172     3.4%

Halfspace.intersectionTime              25,501     0.9%

Vector.__add__                          20,000     0.7%

Vector.isPoint                          20,000     0.7%

addColours                              15,999     0.6%

Scene.rayColour                         15,333     0.5%

<listcomp>                              15,265     0.5%

TOTAL                                2,882,307

objects allocated per render (original):

Vector                                 452,943

Point                                    5,345

Ray                                     98,172

Bottlenecks identified

B1  Type-check calls: 818,411 calls per render (28% of all Python calls)

to mustBeVector()/isPoint(), which do nothing for valid input.

B2  Temporary objects: 452,943 Vector objects per render, most of them

intermediates of expressions like (centre - origin).dot(direction) that

live for a single operation; each needs an allocation, a __dict__ and a

deallocation.

B3  Redundant work in the shadow test: _lightIsVisible() is called 10,666

times but builds a new, identical Ray (sqrt + normalize) for every

object it tests, so the render creates 98,172 Rays although only

~25,999 distinct rays are needed.

B4  Attribute access through per-instance __dict__ on every .x/.y/.z read

and write.

Flame graph - raytrace, original (perf record -e cpu-clock -g, python3-dbg). Interactive version: benchmarks/raytrace/flamegraphs/flamegraph_raytrace_original.svg

# 3. Optimizations

All changes are in benchmarks/raytrace/optimized/run_benchmark.py. The scene,

image size, algorithm, public classes/methods and the benchmark name are

unchanged, so it runs under pyperformance through a custom manifest

(benchmarks/raytrace/manifest/MANIFEST). Every floating-point expression keeps

the original operand order, so the rendered image is BIT-IDENTICAL.

O1  __slots__ on Vector, Point and Ray  (targets B4, B2)

Attributes live at fixed offsets in the object (slot descriptors) instead

of a per-instance dict: faster .x/.y/.z access and cheaper creation.

O2  Remove type-check calls from the hot path  (targets B1)

mustBeVector()/isPoint() calls are dropped; where the result type really

depends on the argument (Vector + Point, Point - Point) a direct class

comparison (other.__class__ is Point) replaces the method call.

O3  Loop-invariant code motion in the shadow test  (targets B3)

_lightIsVisible() builds Ray(p, light - p) once per light instead of once

per tested object. The ray does not depend on the object, so the result

is identical.

O4  Inline hot vector math on floats  (targets B2)

Sphere/Halfspace.intersectionTime, normalAt, Ray construction

(normalization), pointAtTime, reflectThrough, the primary-ray direction

in render() and the Lambert term in colourAt() compute directly on local

floats instead of chaining Vector method calls that create temporaries.

The closest-hit search no longer builds a list of tuples.

Correctness (verify_raytrace.py)

image 100x100 (30000 bytes)

sha256 original : 11f2c36b02044f115c3937cae1156e9528b95da70f9d6b737a262f06757c2e6b

sha256 optimized: 11f2c36b02044f115c3937cae1156e9528b95da70f9d6b737a262f06757c2e6b

differing bytes : 0

ablation stage v0_original   : identical

ablation stage v1_no_checks  : identical

ablation stage v2_hoist      : identical

ablation stage v3_inline     : identical

ablation stage v4_slots      : identical

PASS (bit-identical)

Effect on Python calls and allocations per render

function                              original   optimized

Vector.mustBeVector                    520,539           0

Vector.__init__                        452,943      57,338

Vector.dot                             509,871           0

Sphere.intersectionTime                179,457     179,457

Point.__sub__                          277,865      14,524

Point.isPoint                          277,865           0

Vector.scale                           149,743           0

Ray.__init__                            98,172      26,000

Vector.normalized                      109,887           3

Vector.magnitude                       109,887           0

Halfspace.intersectionTime              25,501      25,501

addColours                              15,999      15,999

Scene.rayColour                         15,333      15,333

Scene._lightIsVisible                   10,666      10,666

Vector.__add__                          20,000           0

Vector.isPoint                          20,000           0

Canvas.plot                             10,000      10,000

<listcomp>                              15,265           0

TOTAL calls                          2,882,307     392,204   (-86%)

Vector objects                         452,943      57,338   (-87%)

Point objects                            5,345       5,345   (+0%)

Ray objects                             98,172      26,000   (-74%)

The amount of real ray-tracing work is unchanged (Sphere.intersectionTime

is called 179,457 vs 179,457 times); what disappears is the call and

allocation overhead around it.

Considered and not pursued

- numpy vectorization: each operation works on a single 3-vector per ray

and the ray tree is recursive and data-dependent, so numpy's per-call

overhead would dominate (as measured for nbody), and vectorizing would

change the floating-point evaluation order (no bit-identical image).

- Changing the scene, image size or recursion depth: would change the

workload, not optimize it.

# 4. Performance comparison

pyperformance (results/compare.txt)

original : 797 ms +- 5.8 ms

optimized: 279 ms +- 2.8 ms

-> 2.85x faster, run time -65%  (meets the >= 7% requirement)

Same trend under python3-dbg while profiling: 2175 ms -> 626 ms (3.48x).

Ablation (ablation_raytrace.py) -- optimizations added one at a time

stage (cumulative)                 time / render  vs original  this step

original                           806.4 +-   8 ms      1.00x      1.00x

+ O2 remove type-check calls       737.1 +-   4 ms      1.09x      1.09x

+ O3 hoist shadow ray              566.1 +-   4 ms      1.42x      1.30x

+ O4 inline vector math            307.9 +-   2 ms      2.62x      1.84x

+ O1 __slots__ (final)             277.7 +-   2 ms      2.90x      1.11x

Every stage renders a bit-identical image (checked in verify_raytrace.py).

perf stat before/after (5 renders per run, mean of 5 runs)

original       optimized   change

instructions        25,641,141,365   9,364,171,653   -63.5%

branches             4,945,496,813   1,764,501,736   -64.3%

branch-misses           14,637,980       8,804,429   -39.9%

cache-misses                34,366          31,698   -7.8%

page-faults                  2,836           2,921   +3.0%

task-clock               4061.4 ms       1472.6 ms   2.76x faster

elapsed                    4.104 s         1.478 s   2.78x faster

Instructions drop by 63% while Python calls drop by 86% and Vector

allocations by 87%. The instruction ratio (2.74x) and the time ratio

(2.76x) agree: the CPU simply executes far less interpreter work for

the same image.

perf profile before/after (flame graphs: flamegraphs/flamegraph_raytrace_*.svg)

Estimated absolute cost = self% x total sampled time of each profile.

role                                             original optimized  change

bytecode dispatch (_PyEval_EvalFrameDefault)        26.6%     28.8%    -60%

Python function calls / frames                      16.3%      7.9%    -82%

attribute access (getattr/setattr, dict, slots)     14.7%      6.7%    -83%

object creation / destruction                        5.6%      6.6%    -57%

float objects: arithmetic + alloc/free               4.0%      7.5%    -31%

math: sqrt / libm                                    0.0%      0.0%     n/a

number-protocol dispatch                             3.4%      4.9%    -46%

tuple/list handling                                  3.5%      3.1%    -67%

debug-build self-checks (python3-dbg only)           6.3%      7.3%    -57%

Largest absolute reductions: _PyEval_EvalFrameDefault (-60%),

call_function.lto_priv.0 (-78%), frame_dealloc.lto_priv.0 (-79%),

_PyEval_MakeFrameVector (-81%), _PyDict_GetItemHint (-90%),

_PyType_Lookup (-80%).

Top of the optimized profile: _PyEval_EvalFrameDefault (28.8%),

binary_op1 (3.0%), PyFloat_FromDouble (1.9%), call_function.lto_priv.0

(1.9%), PyTuple_GetItem (1.7%).

Total sampled time: original 81.20 s, optimized 30.05 s (cpu-clock event

count, includes pyperformance start-up).

Flame graph - raytrace, BEFORE optimization

Flame graph - raytrace, AFTER optimization. Interactive version: benchmarks/raytrace/flamegraphs/flamegraph_raytrace_optimized.svg

# 5. Hardware acceleration proposal

One accelerator serves both benchmarks: V3A, a 3D Vector-math Accelerator.

The hot arithmetic of nbody (pair distance, |d|^-3) and of raytrace

(ray-sphere test: dot products and a square root) is the same small set of

operations: 3D subtract, dot product and inverse square root. V3A implements

them once as reusable pipelined IP and exposes two kernels on one PCIe card.

Full RTL: hw/rtl/*.v (Verilog-2005/SystemVerilog-compatible, Icarus-tested).

## 5.1  Hardware description (hw/rtl)

v3a_top.v             PCIe core logic: register bus decode, IRQ

v3a_nbody_ctrl.v      nbody kernel: body registers, step FSM, integrator

v3a_pair_pipe.v       pairwise gravity datapath, 19 stages

v3a_ray_ctrl.v        ray kernel: ray BRAM, feed FSM, reducer, results

v3a_raysphere_pipe.v  ray-sphere datapath, 19 stages

v3a_invsqrt.v         shared inverse-square-root IP, 12 stages

v3a_delay.v           sideband pipeline balancing

v3a_common.vh         Q23.40 fixed-point multiply

v3a_invsqrt_lut.vh    64-entry initial-guess ROM (generated)

Golden model: hw/model/v3a_model.py (bit-accurate). Testbench:

hw/tb/tb_v3a.v. Everything is run by: python3 hw/run_hw_tests.py

## 5.2  Inputs and outputs

Number format : signed Q23.40, 64 bits (range +-8.4e6, LSB 9.1e-13).

Chosen instead of IEEE-754 double: a fixed-point multiply

needs no normalization/rounding logic (about half the area

of a double-precision FPU multiplier, one stage per op).

Clock         : single clock, target 100 MHz (see 5.7), sync. reset.

Host bus      : memory-mapped, word-addressed, 64-bit data (PCIe BAR0

behind an AXI4-Lite bridge; BAR byte offset = 8 x index).

24-bit address: 0x000000 nbody window, 0x100000 ray window.

Interrupt     : irq (level) when a job is done -> MSI.

nbody kernel  : in : NBODIES (2..8), NSTEPS, DT, per body x,y,z,vx,vy,vz,m*dt

out: updated x,y,z,vx,vy,vz after NSTEPS steps; STEPDONE,

CYCLES (performance counter)

ray kernel    : in : NRAYS (1..4096), NSPH (1..16), EPS, MODE

(0 closest hit, 1 any hit), spheres c,r^2,

rays p,d (d normalized)

out: per ray (sphere index or -1, t)  /  (occluded 1/0)

Pipeline IP   : valid/data streaming, no back-pressure needed (the

controllers always accept every output).

v3a_invsqrt        in  x (Q23.40)                     out y = x^-1/2

v3a_pair_pipe      in  r_i, r_j, m_i*dt, m_j*dt, i, j out f_i, f_j, i, j

v3a_raysphere_pipe in  p, d, c, r^2, ray id, sph id   out disc, t, ids

## 5.3  Hardware architecture

Datapath (see hw/docs/v3a_pipelines.png):

- Every stage has at most ONE 64x64 multiply followed by a register, so

the critical path is the same everywhere and throughput is one result

per clock cycle after the 19-cycle latency.

- v3a_invsqrt: stage 1 normalizes x = m*4^k with m in [1,4) using a

priority encoder and a barrel shift; stage 2 takes y0 from a 64-entry

ROM indexed by the top 6 bits of m; stages 3-11 run three Newton

iterations y <- y(3 - m y^2)/2 (3 stages each: y^2, m*y^2, update);

stage 12 shifts back by k. Measured accuracy: relative error < 2.3e-7

over 1e-6..1e6 (limited by the Q23.40 LSB), ~1e-12 for typical values.

- Sideband data (indices, d, masses, v) travel through v3a_delay shift

registers so they stay aligned with the invsqrt result.

Control logic:

- v3a_nbody_ctrl FSM per time step: CLR (zero accumulators) -> FEED (one

pair (i<j) per cycle into the pipeline) -> WAIT (outputs accumulate

acc_i -= f_i, acc_j += f_j) -> VEL (v += acc for all bodies) -> POS

(r += dt*v, one body per cycle) -> NEXT (repeat NSTEPS times -> IRQ).

All pairs of a step use the positions from the start of the step, so

pairs are independent and can be streamed through the pipeline.

- v3a_ray_ctrl: a feed FSM walks (ray r, sphere s) and streams one test

per cycle (ray words come from a synchronous BRAM, 1-cycle read); a

reducer after the pipeline keeps the running best hit of the current

ray (t > -EPS and smallest t; or any t > EPS for shadow rays) and writes

the result BRAM when the last sphere of that ray comes out.

Verification (hw/run_hw_tests.py, vectors from the real benchmark code):

nbody  : 5 bodies, 2000 steps, 10 pairs/step

RTL vs golden fixed-point model: BIT-EXACT

energy: float benchmark -0.169071606870, hardware -0.169071606646

relative difference 1.33e-09

max |position diff| vs float benchmark: 9.01e-07

cycles: 76000 = 38.0 per step (10 pair feeds +

19 latency + 5 position updates + control)

ray    : closest hit, primary rays

2500 rays x 7 spheres = 17500 tests

RTL vs golden fixed-point model: BIT-EXACT

closest sphere == float benchmark: 2500/2500 rays

(194 hits), max |t error| 1.12e-09

cycles: 17521 = 1.001 per test (+19 latency, +1 BRAM read)

ray    : any hit, shadow rays

1940 rays x 7 spheres = 13580 tests

RTL vs golden fixed-point model: BIT-EXACT

shadow decision == float benchmark: 1940/1940 rays

(53 occluded)

cycles: 13601 = 1.002 per test (+19 latency, +1 BRAM read)

RESULT: PASS

## 5.4  Hardware/software interface

Driver    : Linux PCIe driver, character device /dev/v3a. mmap() of BAR0

for the registers; pinned DMA buffers for bulk data; MSI

interrupt wakes the waiting process.

Transfers : small state (nbody: 5 bodies x 7 words = 280 bytes) is written

directly through BAR0. Ray batches (48 bytes per ray, up to

4096 rays) are copied by the card's AXI4 DMA master from a

pinned host buffer into the ray BRAM; results are copied back

the same way (the RTL models the BRAM filled via the bus).

API       : Python extension module (C, or ctypes over the ioctl):

v3a.nbody_advance(bodies, dt, steps)

v3a.closest_hits(rays, spheres, eps) -> [(index, t), ...]

v3a.any_hits(rays, spheres, eps)     -> [occluded, ...]

It converts floats <-> Q23.40, fills buffers, writes CTRL,

sleeps until the IRQ and reads the results.

Key rule  : offload granularity. One PCIe round trip costs tens of

microseconds, while one CPython pair/intersection costs

~0.4-0.7 microseconds. Calling the card per pair or per ray

would be 10-100x SLOWER than software. Every job is therefore

a batch: all NSTEPS of nbody in one call; all rays of one

recursion level ("wavefront") in one call for raytrace.

(Same lesson as numpy in section 3: fixed per-call overhead

must be amortized over many elements.)

## 5.5  Acceleration justification (raytrace)

Why a good candidate: Sphere.intersectionTime is the single hottest kernel

(179,457 ray-sphere tests per 100x100 frame). Each test is independent of

the others, branch-free until the final comparison and uses only +, -, *

and one sqrt: ideal for a streaming pipeline. The rest of the frame

(shading, recursion, colour mixing) is irregular, data-dependent control

flow and stays in Python.

Measured share (hw/model/raytrace_offload_share.py, Python 3.10: replaying

all sphere tests of one frame versus rendering the frame): about 41% of

the original and about 48% of the optimized render time.

Software changes: render() becomes wavefront (breadth-first): generate all

primary rays -> v3a.closest_hits() -> for each hit build the shadow rays

(v3a.any_hits) and reflection rays (next closest_hits batch) -> repeat up

to depth 4 -> shade in Python. The floor (Halfspace) test stays in

software (one dot product) and is combined with the sphere result.

Estimate (assumptions: 100 MHz, measured ~1.00 cycle per test in RTL

simulation, ~9 batches per frame, ~26k rays x 48 bytes = 1.2 MB of DMA per

frame at ~3.9 GB/s, ~20 us driver overhead per batch):

hardware per frame = 179,457 tests x 10 ns + DMA + 9 calls  ~ 3 ms

optimized CPython: sphere tests ~48% of the frame

Amdahl limit      : 1 / (1 - 0.48)                           ~ 1.9x

realistic         : ~1.6-1.8x over the optimized software

(batch construction in Python costs back part)

Compared to nbody the gain is much smaller: nbody offloads almost the whole

loop, raytrace only its hottest kernel. Moving Lambert shading and

reflection vectors to hardware as well would raise the offloaded fraction,

at the cost of more area and a less reusable design.

Accuracy: RTL results are bit-exact to the golden model; hit/miss and

closest-sphere decisions matched the double-precision benchmark for every

tested primary and shadow ray (t error ~1e-9). Rare decisions exactly at

t ~ +-EPS could in principle differ from double precision, so the image is

not guaranteed to be bit-identical (unlike the software optimization).

## 5.6  Block diagram

Figure 1 - V3A accelerator: system integration (host, driver, PCIe, both kernels)

Figure 2 - V3A datapaths: nbody pair kernel, ray-sphere kernel, shared inverse-sqrt IP

hw/docs/v3a_system.png    system: host, driver, PCIe, both kernels

hw/docs/v3a_pipelines.png datapaths (ray-sphere pipeline and invsqrt)

Python render() --wavefront batch--> v3a ext --> driver --DMA--> ray BRAM

^                                                             |

|                                                  feed FSM (r, s)

|                                                             v

shading <-- (index, t) <-- result BRAM <-- reducer <-- ray-sphere pipe

in Python     per ray                    closest/any     (19 stages,

1 test/cycle)

## 5.7  Performance / area / power trade-offs

Frequency: as for nbody, one 64x64 multiply per stage; conservative

100 MHz on FPGA, ~250 MHz in 28 nm ASIC.

Area: the ray kernel alone needs 17 multipliers (6 in R2, 9 in invsqrt,

2 more) plus a 4096 x 384-bit ray BRAM and a 4096 x 128-bit result BRAM.

Throughput vs area:

- Replicating the ray pipeline P times (spheres or ray groups in parallel)

gives P tests per cycle, but the Amdahl limit (~1.9x) is already close

to being reached with ONE pipeline (~3 ms vs ~85 ms of remaining Python

work), so extra area buys almost nothing here.

- A smaller, iterative (shared multiplier) ray kernel would still take

only ~25 x 179,457 x 10 ns ~ 45 ms per frame and keep ~1.25x; it is a

reasonable low-area option for this benchmark alone.

- Batch size: larger batches amortize PCIe/driver overhead but increase

latency and host memory; per-ray calls would be ~10x slower than CPython.

Power: few watts while busy on an FPGA, near zero when clock-gated between

batches; since the card is idle most of each frame (~3 ms busy of ~90 ms),

average accelerator power is low, but the CPU still does >90% of the work,

so total system energy per frame improves by roughly the speedup only.

## 5.8  Known simplifications of this RTL (deliberate, and what a real card adds)

- Clock domains: the design uses one clock. A real PCIe card runs the user

logic on its own clock and needs asynchronous FIFOs between the PCIe

core clock and the kernel clock; that CDC logic is not modelled here.

- DMA: the RTL fills the ray BRAM through the register bus (the testbench

acts as the driver). A real card uses an AXI4 master (XDMA/QDMA IP) to

read the pinned host buffer; the register map and the FSMs do not change.

- Ray batch size: the ray BRAM holds 4096 rays, while a 100x100 frame has

10,000 primary rays, so the driver issues ~3 batches per wavefront level.

This is already included in the per-batch overhead of the estimate.

- Fixed-point overflow is not detected: Q23.40 wraps silently above

+-8.4e6. Values in both benchmarks stay far below that (the largest

intermediate is |d|^2 < 3e3), but a production version would either

saturate or raise a status flag, at the cost of extra logic per stage.

- nbody body count is limited to NMAX = 8 (state lives in registers so

every pair read is single-cycle). More bodies would move the state into

BRAM and add a load stage to the pipeline.

- No synthesis or timing closure was run (not required): the frequency

numbers are estimates based on one 64x64 multiply per pipeline stage.

# 6. Conclusion

raytrace is dominated by Python call and object overhead, not by the ray

tracing math. Removing no-op type-check calls, hoisting the loop-

invariant shadow ray, inlining the vector math on floats and using

__slots__ cut Python calls by 86% and gives a 2.85x speedup under

pyperformance with a bit-identical image. What remains is the per-ray

intersection and shading arithmetic (179,457 ray-sphere tests per

frame), which is exactly the regular, data-parallel kernel targeted by

the hardware accelerator in section 5.